Developing an Online SSVEP based BCI System Using Ear-EEG

Yu-Te Wang, Student Member, IEEE, Masaki Nakanishi, Member, IEEE, Simon Lind Kappel, Simons’s advisor, Yijun Wang, Member, IEEE, Chung-Kuan Cheng, Fellow IEEE, Tzyy-Ping Jung, Fellow, IEEE

Abstract— The purpose of this study aims to (1) merge two online signal processing algorithms, filter bank CCA and CCA with training dataset, to (2) demonstrate an online SSVEP-based BCI system using Ear-EEG. Our previous studies have shown the aspect that the Ear-EEG is capable to collect SSVEPs. Even in a well control laboratory environment, however, the low signal to noise ratio of acquired SSVEPs makes related applications hard to implement. In the other hand, filter bank- and CCA with training data set based online target detection algorithms have shown the significant performance improvement in terms of the accuracy and ITR in our previous studies. By using optimal parameters for the filter bank CCA and CCA with training data, we assume the Ear-EEG can be used in an SSVEP-based BCI system. Two subjects participated this SSVEP-based BCI experiment using customized, lightweight, 6 channels earpieces for acquiring SSVEPs from ear canal and auricle. Their task was to input four directions by gazing coded and different phase visual stimulus. The offline classification results showed proposed detection algorithm can reach the accuracy of XX% and XX ITR using 4 sec-long SSVEPs acquired from earpieces. In the online experiment, both subject successfully completed the task of manipulating a 4 targets BCI system with XX ITR and the accuracy of XX%. The result suggested that the ability of using proposed merged algorithm and Ear-EEG to perform the SSVEP-based BCI applications. This study might lead an alternative path to implement practical real-world applications.

INTRODUCTION

Steady-State Visual Evoked Potential (SSVEP)- based Bran Computer Interface (BCI) Systems have gained a lot of attentions due to the high information transfer rate (ITR) and requires almost no training [1]. In most of the applications and fundamental studies, placing electrodes on the top of the occipital site seems reasonable since it’s the closed spot to visual cortex and it’s signal to noise ratio  (SNR) is relatively high. However, constrains of gel usage and long time skin preparation make this paradigm unreliable in most real-world applications.

Several efforts have been made in order to overcome the constrains. Mike et al. [2]. proposed dry and noncontact electroencephalogram (EEG) sensors to acquire SSVEPs without gel usage from hair-covered area on scalp. The electrodes featuring custom integrated, high-impedance analog front-end, fingered contact posts, and active buffering circuitry successfully demonstrated real-world application, i.e. no gel usage.  Lin et al. [3] proposed dry polymer form electrodes fabricated by electrically conductive polymer covered by a conductive fabric suggested an alternative approach to long-term acquire EEGs from non-hair-covered area. Similar effort were also made in Huang et al. [4], as they proposed an active comb-shaped dry electrode avoiding signal attenuation and phase distortion. In sum, dry or non-contact sensors introduced a good solution to overcome the challenges including gel usage and long-time preparation. However, for the ventilator users who suffered quadriparetic surgery, it is not easy to collect SSVEPs from occipital site when they were lying on the bed. Therefore, an alternative approach to perform the data collection is crucial. Recently, Ear-EEG has been evaluated and demonstrated that both auditory and visual evoked responses are possible to be collected in ears across a population of subjects [5]. Low SNR, however, makes the online implementation remaining a challenge. To our best knowledge, no studies have shown online applications using this technology in BCI fields.

In the other hand, canonical correlation analysis (CCA) based signal processing algorithm has been proposed and shown the performance improvement in BCI fields [5][6]. In general, this algorithm projected original (EEGs) and template (sinusoids) data sets into a new space such that the correlation of the two projected matrices, termed canonical variables, is maximized. Recently, studies [8][10] proposed a new usage of CCA. They first replaced the sinusoids data sets by pre-recorded training data. The training data were elicited by coded visual stimulus with various phases and frequency[7]. The results showed the performance increased significantly in either accuracy or ITR. Chen et al. further proposed filter bank CCA [9], in which they used the fundamental and harmonic frequency components to improve the accuracy of SSVEPs. The results showed averaged ITR of 151.18±20.34 bits/min across 10 subjects.

This study aimed to merge two online algorithms, filter bank CCA and CCA with training data, to implement an online SSVEP-based BCI system using Ear-EEG [5][8][9]. Two subjects participated this experiment. The offline classification results showed the accuracy of xx % and ITR XX bits/min. The online accuracy can reach the accuracy of xx % while the ITR is XX bits/min. This study demonstrated the possibility of using Ear-EEG to perform an online BCI system even with low SNR.

Method

  - A.	EEG Recording

The visual stimulus is consisted of 4 coded targets rendering on a 21inch screen with resolution of 600*800, as shown in Fig. 1. Each target is a 3cm*3cm square flickering at white and black contrast. The phase and frequency was also coded in each target. For instance, top target was flickering at 9Hz with phase of 0 degree. The detailed coding info can be found in [8][10][11].

Fig. 4. SSVEPs of raw data (top), power spectrum(middle), and SNR(bottom) from occipital site (black), left ear (blue), and right ear (right) when a subject was gazing at 9Hz visual stimulus.

(a)

（b)

Fig1. Four targets layout of proposed visual stimulus. Top was used to show subject’s input. When the training starts, each target was flickering at coded frequency and phase. A red square is the cue that appears for XX mini seconds to assistant subjects to gaze on it. In online experiment, the red square appears for xx mini seconds to indicate the decision by the system.

Two subjects (1 males and 1 female, average XX years old) with normal eyesight participated training and online experiment. They both signed consent form before running the experiment. One of them had experienced using SSVEP-based system while the other one is naive. The training  was consisted of 20 sessions. Each session has three trials lasted 60 seconds, including 4 second for each target and 1 second for gaze shifting. In other words, each target should be gazed three times in random order in one session. In the training, a target was rendered as red for xx mini second as a cue, subjects were asked to gaze at the target as soon as possible. The monitor was 60cm away in front of subjects. A few minutes break were performed between each session.

The earpiece is a customized 3D printing device with six-conduct mental connectors, as shown in Fig. 2(a). It can collect 4 channel data in canal and two channel data from auricle. All of the electrodes were connected to the cable of amplifier.

In order to have a ground truth, eight channels around occipital site following 10-20 system were simultaneously recorded. All channels were reference to forehead. The amplifier was Biosemi ActiveTwo EEG system (Biosemi, Inc.).

  - B. Signal Processing

This study developed an extended CCA-based method to incorporate SSVEP training data in target identification. Fig. 1 shows the flowchart of the proposed method. In addition to the standard CCA method, the proposed method combined correlations between single-trial SSVEPs and SSVEP reference signals in target identification. For each stimulation frequency, the training SSVEP reference signals X can be obtained by averaging multiple SSVEP trials in a training set. Correlation coefficients between projections of test set X and training reference signals X using CCA-based spatial filters can be used as features. Specifically, the following three weight vectors are used as spatial filters to enhance the SNR of SSVEPs: (1) WX(XX) between test set X and training reference signals X, (2) WX(XY) between test set X and sine-cosine reference signals Y, and (3) WX(XY) between training reference signals X and sine-cosine reference signals Y. The target can be identified by recognizing the training reference signal that maximizes the correlation to X. A correlation vector ρ is defined as follows:

ρ=ρ1ρ2ρ3ρ4=ρXTWX(XY), YTWY(XY)ρXTWX(XX), XTWX(XX)ρXTWX(XY), XTWX(XY)ρXTWX(XY), XTWX(XY)          (4)

where ρa,b indicates the correlation coefficient between a and b. An ensemble classifier can be used to combine features derived from the four methods described above. In practice, the following weighted correlation coefficient ρ is used as the final feature in target identification:

ρ=i=14signρi∙ρi2                                         (5)

where sign() is used to remain discriminative information from negative correlation coefficients between test set X and training reference signals X. The training reference signal that maximizes the weighted correlation value is selected as the reference signal corresponding to the target.

(a)

(b)

Fig. 5. The offline analysis result for (a) subject 1 and (b) subject 2.

Filter bank CCA, in general, decomposes the original data into different sub-band data by applying zero-phase Chebyshev infinite impulse response filter. Each sub-band data then feeds into individual standard CCA to produce a correlation. A weights, defined as

wn= n-a+b								 (1)

where n is an index of the sub-band data, a and b are constants maximizing the classification accuracy in the offline analysis. This weight equation applies to correlations of each standard CCA, finally the maximum correlation indicates the result. The detail explanation can be found in [9].

  - C. Training Paradigm

The training was consisted of 20 sessions. Each session has three trials lasted 60 seconds, including 4 second for each target and 1 second for gaze shifting. In other words, each target should be gazed three times in random order in one session. In the training, a target was rendered as red for xx mini second as a cue,subjects were asked to gaze at the target as soon as possible. The monitor was 60cm away in front of subjects. A few minutes break were performed between each session.

  - D. Online Experiment

This is online.

Results

  - A. 	Offline Analysis

Fig. 3 shows channel correlation of six earpiece’s channels among different targets from subject2 Fig. 3(a) and subject 1 Fig. 3(b). The error bar indicates the standard error among six earpiece channels across all targets. As we can see that there was no significant difference among channels. This indicates the difference of SSVEPs collected from each earpiece are similar. Therefore we average the SSVEPs for the further analysis.

TABLE I. Online free input result.

| Subject |  | Input length | Time (sec) | Accuracy (%) | ITR (bits/min) |
| --- | --- | --- | --- | --- | --- |
| S1 | Trial 1 | 15 | 75 | 80 | 11.53 |
|  | Trial 2 | 15 | 75 | 80 | 11.53 |
|  | Trial 3 | 15 | 75 | 80 | 11.53 |
| S2 | Trial 1 | 13 | 65 | 92.31 | 17.84 |
|  | Trial 2 | 13 | 65 | 92.31 | 17.84 |
|  | Trial 3 | 12 | 60 | 100 | 24 |
| Mean |  |  |  | 87.44±8.62 | 15.71±5.1 |

Fig. 4 shows the averaged SSVEPs collected from occipital (black), left ear (blue), and right ear (red) of one subject gazing at 9Hz visual stimulus. The amplitude from ear SSVEPs are obviously smaller than the SSVEPs acquired from occipital over visual cortex. In power spectrum density, as shown in Fig. 4(b), the dominant peak are all located in 9Hz, and second harmonic also appears in occipital and left/right ears. In general, both subjects have second harmonics and almost no further harmonics so the optimal parameter for the filter bank are 2. These parameters would be used for the simulated online and online analysis. Note that, interference noise also appears in ear SSVEPs (marked as “+”), as addressed in the [12].

  - B. 	Simulated Online and Online Analysis

This study first used leave-one-out cross-validation to estimate the BCI performance, as shown in Fig. 5. The accuracy reaches ~80% for both subjects. The results suggest that accuracy increased as longer testing data used.

Table I shows the online results for two subjects. Both subjects completed online task, in which inputs the four directions three times. Ideally, the input length should be 12. This result suggests that the Ear-EEG can indeed to demonstrate the SSVEP-based BCI applications.

Conclusions and Discussions

  - Challenges including easy-of-use and user friendly play important roles in BCI field. Studies have proposed many approaches in order to overcome the problems by developing practical BCI systems in real-world applications. Dry electrodes, non-contact sensors demonstrate one possibility of overcoming the problems in practical BCI systems. Alternatively, Ear-EEG featuring the un-seen, easy-of-use, and continuously monitoring opens another path to increase the usage in BCI systems. This paper proposed a new algorithm to implement an SSVEP-based BCI system using Ear-EEG. The offline training analysis is used to find the best parameters i.e. sub-band and number of harmonics for the further online experiment. The empirical results show the accuracy can reach XX% and ITR can reach XX bits/min.

Appendix

Appendixes should appear before the acknowledgment.

Acknowledgment

We thank to…

References

- Yijun, W. Ruiping, G. Xiaorong, H. Bo, and G. Shangkai, "A practical VEP-based brain-computer interface," Neural Systems and Rehabilitation Engineering, IEEE Transactions on, vol. 14, pp. 234-240, 2006.”

- Y. M. Chi, W. Yu-Te, W. Yijun, C. Maier, J. Tzyy-Ping, and G. Cauwenberghs, "Dry and Noncontact EEG Sensors for Mobile Brain&#x2013;Computer Interfaces," Neural Systems and Rehabilitation Engineering, IEEE Transactions on, vol. 20, pp. 228-235, 2012.

- CT line “novel polymer sensor”. L. Chin-Teng, L. Lun-De, L. Yu-Hang, I. J. Wang, L. Bor-Shyh, and C. Jyh-Yeong, "Novel Dry Polymer Foam Electrodes for Long-Term EEG Measurement," Biomedical Engineering, IEEE Transactions on, vol. 58, pp. 1200-1207, 2011.

- Yan-Jun, “novel active comb-shaped dry electrode” H. Yan-Jun, W. Chung-Yu, A. M. K. Wong, and L. Bor-Shyh, "Novel Active Comb-Shaped Dry Electrode for EEG Measurement in Hairy Site," Biomedical Engineering, IEEE Transactions on, vol. 62, pp. 256-263, 2015.

- Kidmose “P. Kidmose, D. Looney, M. Ungstrup, M. L. Rank, and D. P. Mandic, "A Study of Evoked Potentials From Ear-EEG," Biomedical Engineering, IEEE Transactions on, vol. 60, pp. 2824-2830, 2013.G. bin, X. Gao, Z. Yan, B. Hong, and S. Gao, “an online multi-channel SSVEP-based”

- YT wang  Y. T. Wang*, Y. Wang*, and T. P. Jung, "A Cell-phone based Brain Computer Interface for Communication in Daily Life", Journal of Neural Engineering, vol. 8, pp. 025018(5pp), 2011.

- Chuan Jia, “Frequency and Phase Mixed Coding in SSVEP-based Brain-Computer Interface”, IEEE TRANS on Biomedical Engineering

- M. Nakanishi, M. NAKANISHI, Y. WANG, Y.-T. WANG, Y. MITSUKURA, and T.-P. JUNG, "A HIGH-SPEED BRAIN SPELLER USING STEADY-STATE VISUAL EVOKED POTENTIALS," International Journal of Neural Systems, vol. 24, p. 1450019, 2014.

- X. Chen, Y. Wang, S. Gao, T. -P, Jung, and X. Gao, “Filter bank canonical correlation analysis for implementing a high-speed SSVEP-based brain-computer interface,” (submitted)

- Xiaogang Chen   C. Xiaogang, W. Yijun, M. Nakanishi, J. Tzyy-Ping, and G. Xiaorong, "Hybrid frequency and phase coding for a high-speed SSVEP-based BCI speller," in Engineering in Medicine and Biology Society (EMBC), 2014 36th Annual International Conference of the IEEE, 2014, pp. 3993-3996.

- Yijun Wang, :: Y. Wang, Y. T. Wang, and T. P. Jung, "Visual stimulus design for high-rate SSVEP BCI," Electronics Letters, vol. 46, pp. 1057-1058, 2010.

- Masaki, : M. Nakanishi, W. Yijun, W. Yu-Te, Y. Mitsukura, and J. Tzyy-Ping, "Integrating interference frequency components elicited by monitor refresh rate to enhance frequency detection of SSVEPs," in Neural Engineering (NER), 2013 6th International IEEE/EMBS Conference on, 2013, pp. 1092-1095.

*Resrach supported by ABC Foundation adㄇ

Y. -T. Wang and C. -K Cheng are with Computer Science and Engineering Department, University of California San Diego (UCSD), San Diego, CA, USA.

M. Nakanishi is with Swartz Center for Computational Neuroscience, Institute of Neural Computation, UCSD, San Diego, CA, USA.

Simon and his advisor are with some school and some address.

Y. Wang and T. -P. Jung is with Swartz Center for Computational Neuroscience, Institute of Neural Computation, and Center for Advanced Neurological Engineering, Institute of Engineering in Medicine, UCSD, San Diego, CA, USA (e-mail: jung@sccn.ucsd.edu; phone: 858-822-7555; fax: 828-822-7556).